Ford TOURNEO CONNECT/GRAND TOURNEO CONNECT – PRELIMINARY Specifications

Fuel CONSUMPTION AND PERFORMANCE

|  |  |  | Fuel consumption l/100km (mpg) |  |  | Performance |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Engine | Power (PS) | CO2 (g/km) | Urban | Extra Urban | Combined | Max Speed (km/h) | 0-100 km/h (secs) | 80-120 km/h** (secs) |
| Tourneo Connect |  |  |  |  |  |  |  |  |
| 1.6 Duratorq TDCi (5-sp man) with FE pack* | 75 | 120 | 4.9 (57.6) | 4.4 (64.2) | 4.6 (61.4) | 145 | 17.8 | 25.3 |
| 1.6 Duratorq TDCi (5-sp man) | 75 | 130 | 5.6 (50.4) | 4.6 (61.4) | 5.0 (56.5) | 145 | 17.8 | 25.7 |
| 1.6 Duratorq TDCi (5-sp man) with FE pack | 95 | 120 | 4.9 (57.6) | 4.4 (64.2) | 4.6 (61.4) | 160 | 14.7 | 21.2 |
| 1.6 Duratorq TDCi (5-sp man) | 95 | 130 | 5.6 (50.4) | 4.6 (61.4) | 5.0 (56.5) | 160 | 14.7 | 21.6 |
| 1.6 Duratorq TDCi (6-sp man) | 115 | 130 | 5.6 (50.4) | 4.6 (61.4) | 4.9 (57.6) | 165 | 13.8 | 15.1 |
| 1.0 EcoBoost (6-sp man) | 100 | 129 | 6.4 (44.1) | 5.1 (55.4) | 5.6 (50.4) | 165 | 14.0 | 20.0 |
| 1.6 EcoBoost (6-sp auto) | 150 | 184 | 10.9 (25.9) | 6.3 (44.8) | 8.0 (35.3) | 173 | 10.9 | - |
| Grand Tourneo Connect |  |  |  |  |  |  |  |  |
| 1.6 Duratorq TDCi (5-sp man) with FE pack | 75 | 121 | 4.9 (57.6) | 4.5 (62.8) | 4.6 (61.4) | 145 | 18.3 | 26.4 |
| 1.6 Duratorq TDCi (5-sp man) | 75 | 130 | 5.6 (50.4) | 4.6 (61.4) | 5.0 (56.5) | 145 | 18.3 | 26.8 |
| 1.6 Duratorq TDCi (5-sp man) with FE pack | 95 | 121 | 4.9 (57.6) | 4.5 (62.8) | 4.6 (61.4) | 160 | 15.1 | 22.0 |
| 1.6 Duratorq TDCi (5-sp man) | 95 | 130 | 5.6 (50.4) | 4.6 (61.4) | 5.0 (56.5) | 160 | 15.1 | 22.4 |
| 1.6 Duratorq TDCi (6-sp man) | 115 | 130 | 5.6 (50.4) | 4.6 (61.4) | 4.9 (57.6) | 165 | 14.1 | 15.6 |
| 1.6 EcoBoost (6-sp auto) | 150 | 184 | 10.9 (25.9) | 6.3 (44.8) | 8.0 (35.3) | 173 | 11.1 | - |

*  Fuel Economy pack includes Auto-Start-Stop, Active Grille Shutter, Smart Regenerative Charging

**  In 5th gear

WEIGHTS AND DIMENSIONS

Weights

|  | Kerbweight (kg)# | Gross Vehicle Mass (kg) | Gross Train Mass (kg) | Max. Towable Mass (braked) (kg) | Max. Towable Mass (unbraked) (kg) |
| --- | --- | --- | --- | --- | --- |
| Tourneo Connect |  |  |  |  |  |
| 1.0 EcoBoost | 1420 | 2010 | 2910 | 1000 | 745 |
| 1.6 EcoBoost | 1469 | 2045 | 2945 | 1000 | 750 |
| 1.6 Duratorq 75 PS | 1458 | 2045 | 2945 | 1000 | 750 |
| 1.6 Duratorq 95 PS | 1458 | 2045 | 3145 | 1200 | 750 |
| 1.6 Duratorq 115 PS | 1467 | 2055 | 3155 | 1200 | 750 |
| Grand Tourneo Connect - 7 seats |  |  |  |  |  |
| 1.6 EcoBoost | 1523 | 2320 | 2920 | 750 | 750 |
| 1.6 Duratorq 75 PS | 1512 | 2300 | 2945 | 800 | 750 |
| 1.6 Duratorq 95 PS | 1512 | 2300 | 3145 | 1000 | 750 |
| 1.6 Duratorq 115 PS | 1521 | 2310 | 3155 | 1000 | 750 |

ØPayload = Gross vehicle mass, less kerb mass. All kerb masses quoted are subject to manufacturing tolerances and are for base models with minimum equipment.

uRepresents the lightest kerbweight assuming full fluid levels and 90% fuel levels, subject to manufacturing tolerances and options, etc, fitted.

Dimensions

|  | Tourneo Connect | Grand Tourneo Connect 5-seat | Grand Tourneo Connect 7-seat |
| --- | --- | --- | --- |
| Overall length | 4418 | 4818 | 4818 |
| Overall width with mirrors | 2137 | 2137 | 2137 |
| Overall height | 1852 | 1845 | 1840 |
| Side door entry width | 612 | 839 | 839 |
| Luggage space width between wheel arches | 1192 | 1149 | 1149 |
| Luggage space width | 1477 | 1477 | 1477 |
| Luggage floor to roof | 1245 | 1234 | 1072 |
| Luggage space length 2 seat mode | 1800 | 2179 | 2179 |
| Luggage space length 5 seat mode | 913 | 1305 | 1264 |
| Luggage space length 7 seat mode | n/a | n/a | 450 |
| Luggage capacity (litres)‡ |  |  |  |
| Luggage Capacity 2 seat mode | 2410 | 2761 | 2620 |
| Luggage Capacity 5 seat mode | 1029 | 1529 | 1287 |
| Luggage Capacity 7 seat mode | n/a | n/a | 322 |

BODY AND CHASSIS

| Body Structure | Computer-optimised, high-efficiency, unitary-welded steel body incorporating rigid occupant cell and front and rear energy-absorbing crumple zones; direct-glazed windshield. |  |  |  |
| --- | --- | --- | --- | --- |
| Passive safety and restraint system elements | Integrated passive safety system featuring: - Driver and passenger airbags plus thorax-protecting side airbags for front occupants - Side curtain airbags for front and second row (Tourneo Connect), Side curtain airbags for front, second and third row (Grand Tourneo Connect). - Three-point safety belts in all positions. Front seat belts are specified with outboard pre-tensioners as standard. - Safety belt reminders for driver and front passenger - ISOFIX child seat attachment points on the 2 outer seats in the second row |  |  |  |
| Corrosion protection | Multi-stage paint and body protection process, including zinc precoating for all relevant exterior panels, optimised dip phosphate coat, electrocoat primer, primer/surfacer and basecoat/clearcoat system, plus comprehensive cavity wax injection, PVC underbody coating and stone chip protection. Thick PVC sealing beads for flanges. Front plastic wheel arch liners, rear textile wheel arch liners, anti scuff strips on inner doorsills. |  |  |  |
| Suspension | Front – Independent MacPherson struts with offset coil spring over gas filled damper units and lower L-arms with optimised front rubber bushings and rear bush mounted on separate reinforced cross-member sub-frame, anti roll bar. |  |  |  |
|  | Rear – Torsion beam rear suspension with coil springs and monotube damper units. |  |  |  |
| Steering | Type – Rack and pinion steering with rack-mounted Electric Power Assisted Steering (EPAS) |  |  |  |
|  | Turning circle (Kerb-to-Kerb) – 11.3m (L1), 12.2m (L2) Turning circle (Wall-to-Wall) – 11.7m (L1), 12.5m (L2) |  |  |  |
|  | Turns lock-to-lock – 2.7 |  |  |  |
| Brakes | Dual circuit, diagonally split, hydraulically operated disc brakes front and rear. Vacuum servo assisted with four-channel ABS and electronic brake distribution (EBD) |  |  |  |
|  | Brake disc dimensions (front/ventilated discs): 300mm diameter (Tourneo Connect) 320mm diameter (Grand Tourneo Connect) |  |  |  |
|  | Brake disc dimensions (rear/solid discs): 280mm diameter |  |  |  |
|  | Modulation: ABS, Traction Control, ESC, EBD, Emergency Brake Assist (EBA), Load Adaptive Control (LAC), Hill Start Assist (HSA), Trailer Sway Control (TSC), Emergency Brake Light (EBL), Torque Vectoring Control (TVC) Optional Active City Stop system |  |  |  |
| Wheels and tyres |  |  |  |  |
| Wheel type | Pressed Steel | Alloy |  |  |
| Wheel size | 6.5 x 16” | 6.5 x 16” |  |  |
| Tyre size | 205/60 R 16 | 205/60 R 16 |  |  |
| Spare wheel and tyre | Full-sized spare or Tyre Mobility Kit (varies by market and vehicle specification) |  |  |  |

PETROL ENGINES

|  |  | 1.0-litre EcoBoost (100PS) |  | 1.6-litre EcoBoost (150PS) |  |
| --- | --- | --- | --- | --- | --- |
| Type |  | Inline three cylinder turbo petrol, direct fuel injection and Ti-VCT, transverse |  | Inline four cylinder turbo petrol, direct fuel injection and Ti-VCT, transverse |  |
| Displacement | cm3 | 999 |  | 1597 |  |
| Bore | mm | 71.9 |  | 79.0 |  |
| Stroke | mm | 82.0 |  | 81.4 |  |
| Compression ratio |  | 10.0:1 |  | 10.0:1 |  |
| Max power | PS (kW) | 100 (74) |  | 150 (110) |  |
|  | at rpm | 6000 |  | 5700 |  |
| Max torque | Nm | 170 |  | 240 |  |
|  | at rpm | 1400-4000 |  | 1600—4000 |  |
| Valve gear |  | DOHC with 4 valves per cylinder, twin independent variable cam timing |  | DOHC with 4 valves per cylinder, twin independent variable cam timing |  |
| Cylinders |  | 3 in line |  | 4 in line |  |
| Cylinder head |  | Cast aluminium |  | Cast aluminium |  |
| Cylinder block |  | Cast iron |  | Cast aluminium |  |
| Camshaft drive |  | Low friction Belt-in-Oil with dynamic tensioner |  | Timing belt with dynamic tensioner |  |
| Crankshaft |  | Cast iron, 6 counterweights, 4 main bearings |  | Cast iron, 4 counterweights, 5 main bearings |  |
| Engine management |  | Bosch MED17 with CAN-Bus and individual cylinder knock control |  | Bosch MED17 with CAN-Bus and individual cylinder knock control |  |
| Fuel injection |  | High pressure direct fuel injection with 6 hole injectors |  | High pressure direct fuel injection with 6 hole injectors |  |
| Emission level |  | Euro Stage 5 |  | Euro Stage 5 |  |
| Turbocharger |  | Continental low inertia turbo |  | Borg Warner KP39 low inertia turbo |  |
| Lubrication system |  | Electronically controlled variable displacement oil pump for improved fuel economy |  | Electronically controlled variable displacement oil pump for improved fuel economy |  |
| System capacity with filter | litres | 4.1 |  | 4.1 |  |
| Cooling system |  | Split cooling system with 2 thermostats |  | Water pump with thermostat and valves |  |
| System capacity incl heater | litres | 5.5 |  | 5.5 |  |
| Transmission |  | Durashift 6-speed (B6) manual |  | 6F35 6-speed automatic transmission |  |
| Gear ratios |  |  |  |  |  |
|  |  | 6th 0.683 5th 0.844 4th 0.780 3rd 1.121 2nd 1.864 1st 3.727 Reverse 3.625 Final Drive 4.27 |  | 6th 0.746 5th 1.000 4th 1.446 3rd 1.912 2nd 2.964 1st 4.584 Reverse 2.943 Final Drive 3.066 |  |

Power Curves

1.0-litre EcoBoost 100PS (74kW)

1.6-litre EcoBoost 150PS (110kW)

DIESEL ENGINE

|  |  | 1.6-litre Duratorq TDCi (95PS) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type |  | Inline four cylinder turbo diesel, transverse |  |  |  |
| Displacement | cm3 | 1560 |  |  |  |
| Bore | mm | 75.0 |  |  |  |
| Stroke | mm | 88.3 |  |  |  |
| Compression ratio |  | 16.0:1 |  |  |  |
| Max power | PS (kW) | 75 (55) | 95 (70) | 115 (85) |  |
|  | at rpm | 3500 | 3600 | 3600 |  |
| Max torque | Nm | 220 | 230 | 270 |  |
|  | at rpm | 1500 | 1500—2000 | 1750—2500 |  |
| Valve gear |  | SOHC with 2 valves per cylinder |  |  |  |
| Cylinders |  | 4 in line |  |  |  |
| Cylinder head |  | Cast aluminium |  |  |  |
| Cylinder block |  | Cast aluminium |  |  |  |
| Camshaft drive |  | Timing belt with dynamic tensioner |  |  |  |
| Crankshaft |  | Drop forged steel, 8 counter- weights, 5 main bearings |  |  |  |
| Engine management |  | Ford Common Rail Diesel Engine Management System |  |  |  |
| Fuel injection |  | Common rail direct fuel inj; 1650 bar injection pressure; 7-hole piezo-electric injectors |  |  |  |
| Emission control |  | Oxidation catalyst, water cooled EGR and standard cDPF |  |  |  |
| Emission level |  | Euro Stage 5 |  |  |  |
| Turbocharger |  | Garrett variable geometry turbocharger |  |  |  |
| Lubrication system |  | Pressure-fed lubrication system with full flow oil filter |  |  |  |
| System capacity | litres | 3.8 with filter |  |  |  |
| Cooling system |  | Water pump with thermostat and valves, with thermal management system |  |  |  |
| System capacity | litres | 5.8 incl heater |  |  |  |
| Transmission |  | Durashift 5-speed (MTX75) manual |  |  | Durashift 6-speed (MMT6) manual |
| Gear ratios |  |  |  |  |  |
|  |  | 5th 0.674 4th 0.865 3rd 1.258 2nd 2.048 1st 3.800 Reverse 3.727 Final Drive 3.56 | 5th 0.674 4th 0.865 3rd 1.258 2nd 2.048 1st 3.800 Reverse 3.727 Final Drive 3.56 (Econetic 3.41) | 6th 0.789 5th 0.943 4th 0.868 3rd 1.194 2nd 1.864 1st 3.583 Reverse 3.615 Final Drive 3.69 |  |

Power Curves

1.6-litre Duratorq TDCi 75PS (55kW)

1.6-litre Duratorq TDCi 95PS (70kW)

1.6-litre Duratorq TDCi 115PS (85kW)

* The stated fuel consumption and CO2 emissions are measured according to the technical requirements and specifications of the European Regulation (EC) 715/2007 as last amended. Results in MPG also correspond to this European drive cycle and are stated in imperial gallons. The results may differ from fuel economy figures in other regions of the world due to the different drive cycles and regulations used in those markets

Note: The data information in this press release reflects preliminary specifications and was correct at the time of going to print. However, Ford policy is one of continuous product improvement. The right is reserved to change these details at any time.

About Ford Motor Company
Ford Motor Company, a global automotive industry leader based in Dearborn, Mich., manufactures or distributes automobiles across six continents. With about 175,000 employees and 65 plants worldwide, the company’s automotive brands include Ford and Lincoln. The company provides financial services through Ford Motor Credit Company. For more information regarding Ford’s products, please visit www.ford.com.

Ford Europe is responsible for producing, selling and servicing Ford brand vehicles in 50 individual markets and employs approximately 47,000 employees at its wholly owned facilities and approximately 69,000 people when joint ventures and unconsolidated businesses are included. In addition to Ford Motor Credit Company, Ford Europe operations include Ford Customer Service Division and 24 manufacturing facilities (15 wholly owned or consolidated joint venture facilities and nine unconsolidated joint venture facilities). The first Ford cars were shipped to Europe in 1903 – the same year Ford Motor Company was founded. European production started in 1911.

Contacts:; Detlef Jenter. Ford of Europe. +49 221 901 8745. djenter@ford.com.